Jinseo Lee

CS 3001

28 January 2026

The Boeing 737 Crashes and System Safety Failures

The plane crash incidents were one of the most feared accidents that could happen to human being as there is a very slight chance that one could survive amid the accident for them mostly present few thousands of miles above the ground level. However, the planes are in fact the safest mode of human transportation statistically, considering the safety record, engineering reliability, or the procedures, compared to others such as the car or the train.

In October 2018, all passengers and the cabin crew on Lion Air flight JT610 were found deceased due to its crash to the Java Sea. Approximately five months after, in March 2019, Flight ET302 lost its connection with the control tower and was found crashed in the Ethiopian countryside, ten seconds after its take-off. Investigations later showed that a flight control software feature played a central role in both accidents, and the key similarity found between these two incidents is that they were both the same plane model, the Boeing 737 MAX 8. While the immediate cause involved a software error, deeper analysis shows that the crashes resulted from a set of system-level failures that closely resemble the types of problems described by Nancy Leveson in her article on the Therac-25 medical device accidents.

The technical cause of the crashes was tied directly to the MCAS, Maneuvering Characteristics Augmentation System, which was a software designed to automatically adjust the airplane’s nose when certain condition was met. (Johnston & Harris, 2019, p5) However, the sensor at its nose oftentimes malfunctioned, triggering MCAS to be activated repeatedly and eventually taking the control out of the pilots from both airplanes. While this software behavior triggered the accidents, investigations showed that the failures extended far beyond a simple bug; Design decisions, testing practices, pilot training choices, and organizational pressures all played critical roles.

One casual factor from Nancy Leveson's article "Medical Devices: The Therac-25" that is relevant to the Boeing accidents is ‘Inadequate Software Engineering Practices ‘. In its case, engineers assumed that software could replace hardware safety mechanisms without introducing new risks. A similar issue occurred with the 737 MAX. Boeing treated MCAS as a small feature rather than as a safety-critical control system. The key similarity between the two cases is the failure to view safety holistically. In both situations, engineers focused on whether the individual components work as intended rather than on how those components could interact in dangerous ways. A difference, however, lies in the regulatory environment. Aviation systems are subject to far more external oversight than medical devices were during the Therac-25 era. Despite this, the system-level thinking Leveson advocates was still not fully applied.

Another noticeable factor that is found in both incidents is ‘Overconfidence in Software’. Therac-25 case involves the lack of skepticism of the engineers who assumed that the software would behave in a predictable manner and relied on its automated behavior. Similarly, Boeing heavily relied on MCAS and merely expected the aircraft to behave within the optimistic range they have predicted while testing. According to Hall and Goelz (2019), this confidence reduced the perceived need for redundancy and clear pilot awareness. The similarity between the two cases is the belief that software would not fail in dangerous ways. The difference is that in aviation, pilots are expected to actively manage failures, while Therac-25 operators had far less ability to intervene. Despite this difference, excessive trust in software played a central role in both accidents.

Third and last casual cause of the Therac-25 case and Boeing incidents is ‘Lack of Defensive Design’. According to Herkert (2020), Boeing chose to modify one of their existing aircraft models, the 737 family, which had a serious design problem that brought MCAS onto the table, rather than designing a new one. In the Therac-25 system, the software lacked internal checks to detect and block unsafe conditions, allowing a single failure to cause severe harm. The key similarity is that both systems allowed single failures to escalate without strong safeguards. The difference is that aircraft systems operate in fast-changing environments, which makes defensive design even more important. In both cases, stronger fault detection and containment could have reduced the consequences of software failure.

Even though the Boeing 737 MAX incidents were deeply rooted from the specific software, MCAS, it resembled and aligned closely with the system failures as described by Nancy Leveson in her report about Therac-25 case. Inadequate software engineering practices, overconfidence in software, and the lack of defensive design all have contributed to the fatal incidents to all the tragedies mentioned above. These factors show that complex systems fail not only because of technical errors, but because of flawed assumptions about how humans, software, organizations, and safety interact.

Reference:

Johnston, P., & Harris, R. (2019). The Boeing 737 MAX Saga: Lessons for Software Organizations. https://embeddedartistry.com/wp-content/uploads/2019/09/the-boeing-737-max-saga-lessons-for-software-organizations.pdf

Herkert, J., Borenstein, J., & Miller, K. (2020). The Boeing 737 MAX: Lessons for Engineering Ethics. Science and Engineering Ethics, 26(6), 2957–2974. https://doi.org/10.1007/s11948-020-00252-y

Hall, J., & Goelz, P. (2019, July 17). Opinion | The Boeing 737 Max Crisis Is a Leadership Failure. The New York Times. https://www.nytimes.com/2019/07/17/opinion/boeing-737-max.html

‌